Free printable from Deepsel · usedeepsel.com/hvac-technician-tool-list · Version: October 2026 · Built from 17 HVAC school tool lists, UA and SMART agreements, current job ads, EPA Section 608 and the A2L guidance from AHRI and the equipment makers. Sources and prices are on the web page. Your employer's policy, your union agreement and state law decide who supplies a tool; this sheet is where the shop writes that down.

# HVAC Technician Tool List

| Technician |  | Start date |  |
| --- | --- | --- | --- |
| Position | ☐ Apprentice ☐ Service tech ☐ Installer ☐ Other: ______ | Truck / van # |  |
| Shop |  | List issued by / date |  |

How to use it. The shop marks Supplied by on every row (T = technician buys and owns it, S = shop supplies it) before the list goes to a new hire. Usually is where most union agreements, contractor surveys, job ads and technicians' own accounts put the tool; Varies means they split. Yr: 1 = school programs require it in the first term, 2 = programs schedule it later, a dash = shop equipment. A2L: Yes = the tool must be rated for the mildly flammable refrigerants R-454B and R-32. The technician ticks Have and writes the brand and serial number, which is the personal tool list a union agreement asks for if tools are stolen.

### A. Hand tools and safety gear (the apprentice's first year)

| Have | Tool | Size / spec | Yr | A2L | Usually | Supplied by | Brand, serial, notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | Multi-bit screwdriver (11-in-1 type) with Schrader bit |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Thermostat screwdriver, precision screwdrivers |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Phillips and flat screwdrivers | #1, #2; 1/4 in flat | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Nut drivers | 1/4 and 5/16 in, magnetic | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Lineman's pliers | 9 in | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Needle-nose pliers | 6 to 8 in | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Diagonal cutters |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Tongue-and-groove pliers | 10 and 12 in | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Wire stripper and crimper |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Adjustable wrenches | 8 in and 10 or 12 in | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Combination wrench set | 1/4 to 3/4 in | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Hex key set | SAE and metric | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Tape measure | 25 ft | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Torpedo level |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Hammer | 16 oz | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Utility knife |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Aviation snips | left and right cut | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Scratch awl, inspection mirror |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Flashlight and headlamp |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Jumper wires, test leads | insulated alligator clips | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Hacksaw | 12 in | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Pipe wrenches | 14 in, two | 2 |  | Varies | ☐ T ☐ S |  |
| ☐ | Tool bag |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Safety glasses | ANSI Z87.1; prescription safety glasses are the exception | 1 |  | Shop (OSHA) | ☐ T ☐ S |  |
| ☐ | Cut-resistant gloves, respirator for attics | N95 or better | 1 |  | Shop (OSHA) | ☐ T ☐ S |  |
| ☐ | Safety-toe boots | often a boot allowance | 1 |  | Tech | ☐ T ☐ S |  |

### B. Meters and test instruments

| Have | Tool | Size / spec | Yr | A2L | Usually | Supplied by | Brand, serial, notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | Clamp meter | volts, amps, ohms, microfarads, temperature | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Non-contact voltage tester |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Pipe clamp thermometers, pocket thermometer | two clamps for superheat and subcooling | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Psychrometer | dry bulb and wet bulb | 2 |  | Tech | ☐ T ☐ S |  |
| ☐ | Manometer | gas pressure and static pressure | 2 |  | Varies | ☐ T ☐ S |  |
| ☐ | Micron gauge |  | 2 |  | Varies | ☐ T ☐ S |  |
| ☐ | Wireless probe set | optional | 2 |  | Varies | ☐ T ☐ S |  |
| ☐ | Combustion analyzer | CO and O2 | 2 |  | Shop | ☐ T ☐ S |  |

### C. Refrigerant tools

| Have | Tool | Size / spec | Yr | A2L | Usually | Supplied by | Brand, serial, notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | Manifold gauges and hoses | R-454B and R-32 scales or library; 800 psi hoses, 4000 psi burst | 2 | Yes | Varies | ☐ T ☐ S |  |
| ☐ | Refrigeration service wrench | with 3/16 and 5/16 in hex adapters | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Tubing cutters | mini, and standard to 1-1/8 in | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Deburring tool |  | 1 |  | Tech | ☐ T ☐ S |  |
| ☐ | Flaring tool | 45 degree, eccentric | 2 |  | Tech | ☐ T ☐ S |  |
| ☐ | Swaging tool, tube bender |  | 2 |  | Tech | ☐ T ☐ S |  |
| ☐ | Valve core removal tools | two | 2 |  | Tech | ☐ T ☐ S |  |
| ☐ | Left-hand thread cylinder adapter | for R-454B and R-32 cylinders | 2 | Yes | Varies | ☐ T ☐ S |  |
| ☐ | Vacuum pump | A2L rated; no gas ballast with A2L | - | Yes | Shop | ☐ T ☐ S |  |
| ☐ | Recovery machine | EPA-certified for the refrigerant; A2L rated | - | Yes | Shop | ☐ T ☐ S |  |
| ☐ | Recovery cylinders | DOT; A2L cylinder: left-hand thread, red band | - | Yes | Shop | ☐ T ☐ S |  |
| ☐ | Electronic leak detector | A2L rated; never a flame-type detector | - | Yes | Shop | ☐ T ☐ S |  |
| ☐ | Refrigerant scale | calibrated | - |  | Shop | ☐ T ☐ S |  |
| ☐ | Nitrogen cylinder and regulator | nitrogen flowing while brazing | - |  | Shop | ☐ T ☐ S |  |
| ☐ | Brazing outfit | air-acetylene or oxy-acetylene | - |  | Varies | ☐ T ☐ S |  |
| ☐ | Hand torch | MAPP or propane | 1 |  | Varies | ☐ T ☐ S |  |
| ☐ | Fire extinguisher | dry powder or CO2, at the charging area | - | Yes | Shop | ☐ T ☐ S |  |

### D. Power tools, ladders and shop equipment

| Have | Tool | Size / spec | Yr | A2L | Usually | Supplied by | Brand, serial, notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | Cordless drill and impact driver |  | 1 |  | Varies | ☐ T ☐ S |  |
| ☐ | Drill bits, step bit, driver bits | 1/4 and 5/16 in hex | 1 |  | Varies | ☐ T ☐ S |  |
| ☐ | Hole saw kit |  | - |  | Varies | ☐ T ☐ S |  |
| ☐ | Reciprocating saw, other large power tools |  | - |  | Shop | ☐ T ☐ S |  |
| ☐ | Shop vacuum |  | - |  | Varies | ☐ T ☐ S |  |
| ☐ | Extension cords | 12 gauge, 50 and 100 ft | - |  | Varies | ☐ T ☐ S |  |
| ☐ | Ladders |  | - |  | Shop | ☐ T ☐ S |  |
| ☐ |  |  |  |  |  | ☐ T ☐ S |  |
| ☐ |  |  |  |  |  | ☐ T ☐ S |  |
| ☐ |  |  |  |  |  | ☐ T ☐ S |  |

### Shop tool policy

| Tool allowance | $ ________ per ☐ month ☐ year ☐ None |
| --- | --- |
| Tool purchase account or payroll deduction | ☐ None ☐ Yes: limit $ ________, repaid $ ________ per paycheck |
| Technician's tools broken on the job | ☐ Shop repairs or replaces ☐ Technician replaces ☐ Shop pays ____ % |
| Shop tools lost or damaged |  |
| Boot allowance | $ ________ per year ☐ None |
| Technician / date | Name: _________________ Signature: _________________ Date: ________ |
| For the shop / date | Name: _________________ Signature: _________________ Date: ________ |

The legal floor. Federal wage law (29 CFR 531.35): the cost of tools the employer requires may not cut into minimum wage or overtime pay in any workweek. California's wage orders: required tools are the employer's, except that an employee earning at least twice the state minimum wage may be required to provide hand tools customarily required by the trade; the rule does not apply to apprentices registered with the state Division of Apprenticeship Standards. OSHA (29 CFR 1910.132(h)): the employer pays for required protective equipment, except non-specialty safety-toe boots and prescription safety glasses the employee may wear off the job. Other state rules may apply.

## Appendix A: Truck Stock Count

| Truck / van # |  | Technician |  |
| --- | --- | --- | --- |
| Count date |  | Counted by |  |

Carry is the quantity on the van at the start of each day. Count what is on hand, and restock the difference. Filled-in Carry quantities are what working technicians report carrying, except the fire extinguisher's, which is from an R-454B installation manual (each one is named on the web page); a blank is yours to set from your own call history. The rule on HomyScan's truck stock list: once you reach for a part more than once or twice in a season, carry two or three. Cylinders: each one 220 lb gross or less and all hazardous materials on the van 440 lb or less (DOT materials of trade, 49 CFR 173.6); R-32 and R-454B are flammable gases for transport and ride upright.

| Item | Sizes / values | Carry | On hand | Restock | Notes |
| --- | --- | --- | --- | --- | --- |
| Electrical |  |  |  |  |  |
| Run capacitor, single, 440 V | 5, 7.5, 10, 15 MFD on Totaline's list; HomyScan adds 20 | 2 |  |  |  |
| Dual run capacitor, 440 V | 30/5, 35/5, 40/5, 44/5, 50/5, 55/5, 60/5, 70/7.5 (Totaline heating list) | 2 |  |  |  |
| Dual run capacitor 45/5 and 45/7.5 |  | 4 |  |  |  |
| Universal multi-tap capacitor (Turbo 200 type) |  | 1 |  |  | Temporary fit until the exact size is in hand |
| Hard start kit |  | 4 |  |  |  |
| Contactor, 24 V coil | 1-pole and 2-pole 30 A and 40 A; 3-pole for light commercial | 3 |  |  |  |
| Fuses, 15 to 50 A, 250 V and 600 V | plus 3 A and 5 A blade fuses for control boards | 3 |  |  |  |
| Transformer | 40 VA, 120/208/240 V primary, 24 V secondary; multi-tap |  |  |  |  |
| Relays | fan relay SPST and SPDT 24 V; DPDT 24 V |  |  |  |  |
| Time-delay relay | delay-on-make, delay-on-break |  |  |  |  |
| Wire nuts, crimp terminals, thermostat wire | terminal kit, twist-on connector kit |  |  |  |  |
| Motors |  |  |  |  |  |
| Condenser fan motor, universal PSC | 1/3 to 1/6 HP, 208-230 V, 825 and 1075 rpm | 2 |  |  |  |
| Blower motor, universal PSC | 1/2 to 1/6 HP and 3/4 to 1/5 HP, 115 V and 230 V | 2 |  |  |  |
| Inducer motor, OEM control board |  |  |  |  |  |
| Gas heat |  |  |  |  |  |
| Hot surface igniter, universal nitride 120 V |  | 2 |  |  |  |
| Flame sensor, universal |  | 5 |  |  |  |
| Gas valve, universal single-stage | 24 V, fast opening, 1/2 x 1/2 (e.g. White-Rodgers 36J22-214) | 2 |  |  |  |
| Gas valve, two-stage | e.g. White-Rodgers 36J54-214 | 1 |  |  | Smart and intermittent-pilot valves: order them |
| Pressure switches | the common ranges in your area |  |  |  |  |
| Limit switches, fan-limit control |  |  |  |  |  |
| Thermocouples, thermopile | 24, 30 and 36 in |  |  |  |  |
| Sequencer (electric heat) | single and double |  |  |  |  |
| Refrigerant circuit |  |  |  |  |  |
| Liquid line filter drier | 8 and 16 cu in, 3/8 in ODF; bi-flow drier for heat pumps |  |  |  |  |
| Schrader cores, port caps, locking caps |  |  |  |  |  |
| Refrigerant: R-410A | 25 lb cylinder |  |  |  | Sale needs a 608-certified buyer |
| Refrigerant: R-454B | 20 lb cylinder, red band, left-hand thread |  |  |  | Division 2.1 for transport: carry upright |
| Refrigerant: R-32 | 20 lb cylinder, red band, left-hand thread |  |  |  | Division 2.1 for transport: carry upright |
| Nitrogen |  | 2 |  |  | Swap the empty one at the supply house |
| Recovery cylinder | A2L-rated cylinder for R-454B and R-32 | 2 |  |  |  |
| Left-hand thread cylinder adapter (A2L) |  |  |  |  | On both Totaline truck stock lists |
| Brazing rod and flux | 15% silver |  |  |  |  |
| Oil | POE, mineral, vacuum pump oil |  |  |  |  |
| TXV |  |  |  |  |  |
| Controls |  |  |  |  |  |
| Thermostats | single-stage and two-stage heat/cool |  |  |  |  |
| Float switch |  | 1 |  |  |  |
| Condensate |  |  |  |  |  |
| Condensate pump | 115 V, 230 V, mini-split |  |  |  |  |
| PVC drain fittings, traps, pan tablets | 3/4 in |  |  |  |  |
| Filters and cleaning |  |  |  |  |  |
| Air filters | 1, 2 and 4-5 in media in your area's common sizes |  |  |  |  |
| Coil cleaner | no-rinse evaporator foam |  |  |  |  |
| Supplies and safety |  |  |  |  |  |
| Screws, tape, sealant, zip ties |  |  |  |  |  |
| Fire extinguisher, dry powder or CO2 | near the charging area on A2L jobs | 1 |  |  |  |

## Appendix B: Shop Tool Issue Record

One sheet per technician. List every shop-owned tool on the truck or in the bag, with the serial number, when it is issued and when it comes back. The Southern California UA service agreement lets the employer require a signed receipt for hand tools issued to employees; this sheet is that receipt.

| Technician |  | Truck / van # |  |
| --- | --- | --- | --- |

| Tool | Make / model | Serial # | Issued | Condition | Returned | Tech initials |
| --- | --- | --- | --- | --- | --- | --- |
| Vacuum pump |  |  |  |  |  |  |
| Recovery machine |  |  |  |  |  |  |
| Recovery cylinder |  |  |  |  |  |  |
| Recovery cylinder (A2L) |  |  |  |  |  |  |
| Electronic leak detector |  |  |  |  |  |  |
| Refrigerant scale |  |  |  |  |  |  |
| Nitrogen regulator |  |  |  |  |  |  |
| Micron gauge |  |  |  |  |  |  |
| Manifold gauges |  |  |  |  |  |  |
| Combustion analyzer |  |  |  |  |  |  |
| Ladder |  |  |  |  |  |  |

| Received in working order | Technician: _______________ Signature: _______________ Date: ________ |
| --- | --- |
| Issued by | Name: _________________ Signature: _________________ Date: ________ |

Keep this record with the tool policy above. In California, IWC Order 4 (section 8) bars deductions for lost or broken equipment unless the loss came from a dishonest or willful act or gross negligence. The spreadsheet version of the tool list and the truck stock count, with the restock column worked out for you, is at usedeepsel.com/hvac-technician-tool-list. Every free checklist and template in the set is at usedeepsel.com/checklists.